# INTEGRATED CIRCUITS TECHNOLOGY

# EXERCISE # 4

- A 1-micron technology MOSFET having W/L = 10.

  - Find its channel resistance if its sheet resistance when ON is R = 10 k.

  - Estimate the channel resistivity if its depth is 10 nm.

  - Calculate the gate cap if the gate oxide thickness is 30 nm (r = 4 for SiO2).

  - Draw the AA and Poly masks indicating L and W.

  - Redraw the layout if the transistor is split into two identical ones in parallel.

- Assume that the CLM (channel length modulation) parameter  is proportional to 1/L, show that the drain dynamic conductance in saturation is proportional to 1/L2.

- Extract the nMOS transistor-level view (schematic diagram) from the following physical-level view (layout diagram).

VDD

A

B

VSS

VDD